# 基于守恒传热传质模型的药材烘干过程分析

# 摘  要

针对圆柱形药材热风烘干过程中的内部温度、水分浓度及烘干时长预测问题，建立径向一维传热传质模型。将题目给定的烘房温湿度和药材半径作为时变输入，通过守恒通量离散与隐式时间积分，求解固定半径和收缩条件下的温度场及干基含水率场。

针对问题一，采用给定常物性参数和随含水率变化的扩散系数，描述预热平衡阶段。1800 s时，中心与表面温度分别为33.5753 ℃和36.7856 ℃，中心与表面水分浓度分别为2.5500 kg/kg和1.5102 kg/kg，水分变化主要集中在外层。

针对问题二，将密度、比热容、热导率和扩散系数更新为题目附录3给定的经验函数，保留变系数通量形式。3 h时，中心与表面温度分别为49.8495 ℃和49.9664 ℃；相应水分浓度为1.7662 kg/kg和1.0081 kg/kg。温度趋于均匀时，药材内部仍存在较大的含水率梯度。

针对问题三，以全体径向节点的最大水分浓度严格低于0.15 kg/kg为终止条件，得到烘干时长57.1697 h。针对问题四，以归一化材料坐标处理均匀径向收缩，结合附录4经验公式，得到烘干时长50.8244 h，终点半径为1.2000 cm。两种情形的差异同时包含几何变化与物性变化的影响。

计算采用321个径向节点，并对水分扩散系数沿浓度积分，以减轻低含水率区域的通量近似误差。161与321节点所得两种烘干时长均相差不足2 s。模型适用于所列一维、有效扩散及均匀收缩假设；表中四位小数是输出格式，不代表所有场值均具有同等精度。

关键词：药材烘干；径向传热传质；非线性扩散；有限体积法；材料坐标

# 一、问题重述

中药材热风烘干通过调节烘房温湿环境，使热量由表面传入药材，并使内部水分向外迁移。题目给定药材近似为长25 cm、初始半径2 cm的圆柱体，初始温度28 ℃，初始干基含水率2.55 kg/kg。附件1提供烘房温度和水分浓度随时间的变化，附件2提供药材半径随时间的变化[1]。

需要解决四个相互衔接的问题：首先，预测预热平衡阶段1800 s内的温度和水分浓度；其次，采用随温度和含水率变化的物性参数，给出3 h内的分布；再次，确定药材各处水分浓度均低于0.15 kg/kg所需的时间；最后，考虑尺寸收缩与另一组经验物性公式，重新确定烘干时长。六张指定时空截面表在相应求解小节中给出，完整结果按题目规定的时间与空间间隔导出。

# 二、问题分析

圆柱体的半径远小于长度，在忽略端面影响的近似下，可将三维传热传质问题简化为径向一维问题。径向温度梯度决定热通量，含水率梯度决定扩散通量，药材表面通过对流与烘房交换热量和水分。温度改变扩散系数，含水率改变热物性，因此问题二至问题四需要同步更新两个场。

问题一主要考察升温初期的瞬态响应。问题二将物性参数与状态关联，问题三在同一模型上延长积分并搜索达标事件。由于终点条件要求“各处”达标，判断对象应为整个径向计算域上的最大值，不能只检查表面或平均水分浓度。问题四还需建立材料点在收缩前后的对应关系，并将固定距离处的输出与移动表面处的输出区分。

在低含水率区域，扩散系数中的负指数项会形成较大的扩散阻力。为避免粗网格下对该阻力的误判，先以界面通量构造离散方程，再以扩散系数的浓度积分计算水分通量。时间积分采用适合刚性方程的BDF方法，计算步长与结果保存间隔分别设置。

# 三、模型假设与符号说明

## 3.1 模型假设

（1）药材近似为均匀圆柱体，初始温度和水分浓度处处相同；忽略轴向输运和端面效应，只考察径向分布。

（2）将题目中的干基含水率视为有效扩散模型的状态变量。题目给出的密度用于热容计算，不额外据此重建空间变化的干固体质量密度。

（3）采用题目给定经验物性公式；问题二至问题四沿用对流换热系数25 W/(m²·K)和对流传质系数8×10⁻⁷ m/s，因为附录3、4未给出替代值。

（4）烘房温度、水分浓度和半径数据在相邻观测时刻之间线性插值。超出观测范围时保持末值，环境输入在4 h之后保持50.165 ℃和0.04986 kg/kg。

（5）热方程忽略蒸发潜热、化学反应热及收缩机械功。计算结果是这些简化条件下的温度预测，不直接用于判断有效成分保留率或能耗。

（6）问题四假设径向收缩均匀，材料点的归一化半径保持不变；给定半径轨迹与场变量共同构成有效模型，不另行假定收缩率与含水率的本构关系。

## 3.2 主要符号

表A  主要符号与单位

| 符号 | 含义 | 单位 |
| --- | --- | --- |
| t | 时间 | s |
| r，R(t) | 径向位置与药材表面半径 | m |
| x=r/R(t) | 归一化材料坐标 | 1 |
| T，TK | 摄氏温度与热力学温度 | ℃，K |
| C | 药材干基含水率 | kg/kg |
| T∞，C∞ | 烘房温度与水分浓度 | ℃，kg/kg |
| ρ，cp | 密度与比热容 | kg/m³，J/(kg·K) |
| k，D | 热导率与水分扩散系数 | W/(m·K)，m²/s |
| hT，hm | 对流换热与传质系数 | W/(m²·K)，m/s |
| N | 径向网格区间数 | 1 |
| Vi | 去除公共几何因子后的控制体体积 | m² |
| Fi+1/2 | 面向径向外侧的水分通量乘以界面半径 | m²/s |
| t* | 首次严格达标的模型时间 | s |

表中浓度单位沿用题目定义；水分通量属于有效浓度方程的通量，不作为实际蒸发质量流率直接使用。摄氏温度与开尔文温度通过 TK=T+273.15 转换。

# 四、通用模型与数值方法

## 4.1 数据插值和边界条件

附件1共有241个观测点，时间间隔为60 s，覆盖0至14400 s；附件2共有145个半径观测点，覆盖0至259200 s。对任一输入变量 u，在相邻时刻之间采用下式插值：

u(t)=uj+t−tjtj+1−tj(uj+1−uj),tj≤t≤tj+1	(4.1)

图1  烘房环境输入与药材半径观测

初始时刻的药材状态为

T(r,0)=28,C(r,0)=2.55	(4.2)

中心满足轴对称条件；在表面，以沿径向外侧为正方向，采用对流换热与传质边界：

∂T∂r|r=0=0,∂C∂r|r=0=0	(4.3)

−k∂T∂r|r=R=hT(Ts−T∞)	(4.4)

−D∂C∂r|r=R=hm(Cs−C∞)	(4.5)

其中下标 s 表示表面。表面温度不必等于烘房温度，表面水分浓度也不直接赋值为烘房水分浓度，而是由内部输运与边界交换共同确定。

## 4.2 径向传热传质方程

在固定半径条件下，根据径向热通量和有效扩散通量的平衡关系，控制方程写为[2]

ρ(C)cp(C)∂T∂t=1r∂∂rrk(C)∂T∂r	(4.6)

∂C∂t=1r∂∂rrD(TK,C)∂C∂r	(4.7)

当系数随含水率变化时，必须将其保留在空间导数内部。只有系数为常数时，才能将其提出导数并写为系数乘以柱坐标拉普拉斯算子。问题一的热导率为常数，但扩散系数仍随含水率变化，浓度方程同样使用通量形式。

## 4.3 控制体离散与非线性水分通量

将半径区间划分为 N=320 个网格区间，包含中心与表面的321个节点。控制体界面取相邻节点的中点，中心和表面采用半控制体。去除公共因子后，各控制体的几何权重为

Vi=ri+1/22−ri−1/222	(4.8)

在界面处计算统一通量，使同一内部界面对相邻控制体的贡献大小相等、方向相反。热导率取相邻节点值的调和平均。水分方程的离散形式为

VidCidt=Fi−1/2−Fi+1/2	(4.9)

对于题目给定的扩散系数，可以统一写成 D=A(TK)exp(−b/C)。定义关于浓度的原函数

Φb(C)=Cexp(−b/C)+bEi(−b/C),Φb′(C)=exp(−b/C)	(4.10)

其中 Ei 为指数积分函数。在一个网格界面附近，以相邻节点平均温度计算 A，水分通量近似为

Fi+1/2=−ri+1/2A(TK,i+1/2)Φb(Ci+1)−Φb(Ci)ri+1−ri	(4.11)

这一写法对浓度依赖部分进行积分，避免在表面浓度较低、扩散系数跨越较大范围时，简单平均系数造成过大的局部阻力误差。它仍是原有效扩散方程的离散近似，温度在界面内的变化通过网格加密控制。中心通量为零，表面通量由对流条件直接给定。

## 4.4 时间积分与结果输出

空间离散后得到温度与水分浓度组成的常微分方程组。使用 SciPy 的隐式BDF积分器[3]，提供相邻节点耦合的稀疏雅可比结构；相对容差设为10⁻⁷，温度和含水率绝对容差分别为10⁻⁸和10⁻¹⁰，最大内部时间步为60 s。每次计算右端项时，根据当前状态更新物性参数和环境边界。

积分器自适应选择内部步长，利用连续插值输出题目规定的时刻。问题一和问题二每隔1 s输出，问题三和问题四每隔60 s输出，并额外加入烘干结束时刻。所有问题均在物理半径每隔0.1 cm处输出；问题四超出当前半径的位置留空，不将域外点赋零。初始行直接保存给定初值。

计算过程不采用裁剪负浓度或降低扩散系数的松弛方式掩盖不稳定。终止条件使用未舍入状态计算，保存到表格时再按四位小数显示。数值结果的收敛性在第9节独立检验。

# 五、问题一预热平衡阶段

## 5.1 物性参数与求解设置

药材半径固定为0.02 m。密度、比热容和热导率分别为820 kg/m³、2600 J/(kg·K)和0.36 W/(m·K)，水分扩散系数按题目附录2取

D(C)=7×10−9exp0.89C	(5.1)

指数中的含水率位于分母。该系数随含水率下降而减小，因此外层变干会提高后续水分迁移的阻力。将上述系数代入通用模型，从初始状态积分至1800 s，得到表1和表2。

表1  30分钟内药材的温度  单位 ℃

| 时间 / s | 0 cm | 0.5 cm | 1 cm | 1.5 cm | 2 cm |
| --- | --- | --- | --- | --- | --- |
| 100 | 28.0001 | 28.0003 | 28.0040 | 28.0327 | 28.1801 |
| 300 | 28.0408 | 28.0635 | 28.1514 | 28.3681 | 28.8488 |
| 600 | 28.4533 | 28.5360 | 28.8039 | 29.3159 | 30.1652 |
| 900 | 29.3243 | 29.4582 | 29.8755 | 30.6161 | 31.7304 |
| 1200 | 30.5427 | 30.7098 | 31.2224 | 32.1126 | 33.4276 |
| 1500 | 31.9957 | 32.1867 | 32.7660 | 33.7463 | 35.1204 |
| 1800 | 33.5753 | 33.7720 | 34.3642 | 35.3621 | 36.7856 |

表2  30分钟内药材的水分浓度  单位 kg/kg

| 时间 / s | 0 cm | 0.5 cm | 1 cm | 1.5 cm | 2 cm |
| --- | --- | --- | --- | --- | --- |
| 100 | 2.5500 | 2.5500 | 2.5500 | 2.5500 | 2.2472 |
| 300 | 2.5500 | 2.5500 | 2.5500 | 2.5492 | 2.0509 |
| 600 | 2.5500 | 2.5500 | 2.5500 | 2.5353 | 1.8770 |
| 900 | 2.5500 | 2.5500 | 2.5497 | 2.5045 | 1.7547 |
| 1200 | 2.5500 | 2.5500 | 2.5482 | 2.4646 | 1.6586 |
| 1500 | 2.5500 | 2.5499 | 2.5445 | 2.4206 | 1.5787 |
| 1800 | 2.5500 | 2.5497 | 2.5383 | 2.3755 | 1.5102 |

## 5.2 结果分析

在1800 s时，中心温度为33.5753 ℃，表面为36.7856 ℃，表面比中心高3.2102 ℃。随着烘房升温，热量由表面逐步传向中心，温度分布保持连续的径向梯度。同期中心水分浓度仍约为2.5500 kg/kg，表面已降至1.5102 kg/kg。

与温度相比，水分变化集中在靠近表面的区域，说明预热阶段内部水分的迁移尚不充分。中心浓度按四位小数显示仍为2.5500，并不意味着其在数学上严格恒定，而是变化量尚未反映在该显示精度上。

# 六、问题二变物性烘干过程

## 6.1 经验物性与耦合关系

问题二从题目给定初始状态出发，在整个0至3 h区间统一采用附录3经验公式：

ρ=650+128C,cp=1450+2736CC+1	(6.1)

k=0.21+0.38CC+1	(6.2)

D=2.4×10−3exp0.45Cexp3850TK	(6.3)

题目要求该问统一采用附录3，因此不在0.5 h时强行拼接问题一的常物性解。问题二与问题三使用相同的初始条件、经验公式和边界输入，问题二的结果可以直接取问题三完整积分在0至3 h内的部分。

表3  3小时内药材的温度  单位 ℃

| 时间 / h | 0 cm | 0.5 cm | 1 cm | 1.5 cm | 2 cm |
| --- | --- | --- | --- | --- | --- |
| 0.5000 | 32.1893 | 32.3821 | 32.9659 | 33.9608 | 35.4131 |
| 1.0000 | 40.3816 | 40.5536 | 41.0605 | 41.8782 | 42.9974 |
| 1.5000 | 45.8468 | 45.9348 | 46.1930 | 46.6051 | 47.1401 |
| 2.0000 | 48.4502 | 48.4881 | 48.5984 | 48.7736 | 49.0034 |
| 2.5000 | 49.4670 | 49.4792 | 49.5136 | 49.5653 | 49.6607 |
| 3.0000 | 49.8495 | 49.8553 | 49.8746 | 49.9101 | 49.9664 |

表4  3小时内药材的水分浓度  单位 kg/kg

| 时间 / h | 0 cm | 0.5 cm | 1 cm | 1.5 cm | 2 cm |
| --- | --- | --- | --- | --- | --- |
| 0.5000 | 2.5499 | 2.5489 | 2.5256 | 2.3257 | 1.6486 |
| 1.0000 | 2.5257 | 2.4948 | 2.3578 | 2.0230 | 1.4711 |
| 1.5000 | 2.3861 | 2.3256 | 2.1344 | 1.8020 | 1.3476 |
| 2.0000 | 2.1709 | 2.1084 | 1.9236 | 1.6259 | 1.2311 |
| 2.5000 | 1.9566 | 1.9006 | 1.7360 | 1.4720 | 1.1166 |
| 3.0000 | 1.7662 | 1.7165 | 1.5702 | 1.3333 | 1.0081 |

## 6.2 结果分析

3 h时中心与表面温差为0.1169 ℃，温度场已经较为均匀；中心含水率仍为1.7662 kg/kg，较初始值下降30.74%，与表面之间仍相差0.7581 kg/kg。温度接近均匀并不能作为烘干结束的依据。

图2  问题一和问题二中中心与表面的状态变化

扩散系数同时受到温度升高与含水率降低的影响。升温使Arrhenius温度因子增大，促进扩散；含水率降低使浓度因子减小，延缓后期干燥。两种影响随时间相互竞争，因而不能用恒定速率线性外推最终烘干时长。

# 七、问题三烘干终止时间

## 7.1 终止判据

药材各处的水分浓度均需严格低于0.15 kg/kg。将连续空间条件在计算网格上进行检查，定义

g(t)=max0≤i≤NCi(t)−0.15	(7.1)

先检测 g(t) 由正向负穿越零点的连续事件，再向上取下一个整数秒作为输出终点；同时核对前一个整数秒仍未满足严格不等式。该判据没有预设最大含水率一定出现在中心，虽然本次计算中中心确为最晚达标位置。

## 7.2 烘干时长与浓度分布

连续阈值事件发生于205810.4112 s，首个严格达标的整数秒为205811 s，对应57.1697 h。表5给出每隔6 h及结束时刻的水分浓度。

表5  固定半径药材烘干过程的水分浓度  单位 kg/kg

| 时间 / h | 0 cm | 0.5 cm | 1 cm | 1.5 cm | 2 cm |
| --- | --- | --- | --- | --- | --- |
| 6.0000 | 1.0156 | 0.9868 | 0.9006 | 0.7546 | 0.5335 |
| 12.0000 | 0.4548 | 0.4418 | 0.4019 | 0.3289 | 0.1640 |
| 18.0000 | 0.2979 | 0.2906 | 0.2679 | 0.2247 | 0.0842 |
| 24.0000 | 0.2371 | 0.2321 | 0.2161 | 0.1851 | 0.0663 |
| 30.0000 | 0.2053 | 0.2013 | 0.1887 | 0.1637 | 0.0597 |
| 36.0000 | 0.1854 | 0.1821 | 0.1714 | 0.1500 | 0.0565 |
| 42.0000 | 0.1717 | 0.1687 | 0.1593 | 0.1404 | 0.0547 |
| 48.0000 | 0.1615 | 0.1588 | 0.1504 | 0.1332 | 0.0536 |
| 54.0000 | 0.1535 | 0.1511 | 0.1433 | 0.1275 | 0.0528 |
| 57.1697 | 0.1500 | 0.1477 | 0.1402 | 0.1249 | 0.0525 |

终点前一秒的最大水分浓度为0.1500001205 kg/kg，终点为0.1499998274 kg/kg；前者高于阈值，后者低于阈值。表5中的终点中心浓度显示为0.1500，是四舍五入造成的显示结果，达标判定始终使用未舍入值。

前期含水率下降较快，后期随着表面含水率降低，扩散系数显著减小，水分迁移速度逐渐减缓。由中心控制的终止时间明显长于温度均衡所需的时间。4 h以后环境保持末值的假设影响整个后期积分，因此本结果应与该边界设定一并使用。

# 八、问题四考虑尺寸收缩

## 8.1 材料坐标与移动边界

直接在不断缩小的物理区间内截取旧浓度场，会丢弃外层材料点的信息。为保持材料点对应关系，定义归一化坐标 x=r/R(t)，并假设材料径向速度为 v(r,t)=rR˙(t)/R(t)。对任一随材料变化的状态量 f，有

∂f∂t|x=∂f∂t|r+v∂f∂r	(8.1)

因此在固定的材料网格 0≤x≤1 上，控制方程可写为

ρcp∂T∂t|x=1R(t)2x∂∂xxk∂T∂x	(8.2)

∂C∂t|x=1R(t)2x∂∂xxD∂C∂x	(8.3)

在该材料导数与有效干基扩散的建模约定下，不再重复加入网格对流项或将含水率按体积比进行稀释。表面条件经 ∂/∂r=R−1∂/∂x 换算。给定半径用于更新扩散距离与边界几何因子，状态值则始终随相同材料坐标推进。

## 8.2 收缩条件下的物性公式

采用题目附录4的经验物性公式：

ρ=760+90C,cp=1850+2150CC+1	(8.4)

k=0.12+0.20CC+1	(8.5)

D=4.2×10−4exp0.30Cexp3850TK	(8.6)

经验密度随含水率下降而减小。该关系用于热方程的体积热容，不支持“收缩必然使经验密度增加”的推断；扩散系数也没有显式密度项。对收缩影响的解释应同时考虑半径缩小和经验参数变化。

## 8.3 计算结果及比较

问题四的连续阈值事件为182967.6715 s，首个严格达标整数秒为182968 s，即50.8244 h。结束时半径为1.2000 cm，相对初始值减小40.00%。

表6  收缩药材烘干过程的水分浓度  单位 kg/kg

| 时间 / h | 0 cm | 0.5 cm | 1 cm | 药材表面 | 半径 / cm |
| --- | --- | --- | --- | --- | --- |
| 6.0000 | 1.7172 | 1.5357 | 1.0217 | 0.4217 | 1.3740 |
| 12.0000 | 0.7343 | 0.6518 | 0.4069 | 0.1667 | 1.2480 |
| 18.0000 | 0.4066 | 0.3668 | 0.2388 | 0.0889 | 1.2140 |
| 24.0000 | 0.2837 | 0.2599 | 0.1783 | 0.0671 | 1.2040 |
| 30.0000 | 0.2253 | 0.2085 | 0.1488 | 0.0593 | 1.2010 |
| 36.0000 | 0.1921 | 0.1790 | 0.1312 | 0.0558 | 1.2000 |
| 42.0000 | 0.1706 | 0.1598 | 0.1196 | 0.0539 | 1.2000 |
| 48.0000 | 0.1556 | 0.1463 | 0.1113 | 0.0529 | 1.2000 |
| 50.8244 | 0.1500 | 0.1413 | 0.1081 | 0.0525 | 1.2000 |

表6各列表示到药材中心的物理距离；在列出的6 h及以后时刻，半径已小于1.5 cm，因此只列0、0.5、1 cm与真实表面位置。完整结果中仍保留0至2 cm的固定网格，超过当前半径的单元格为空。表面一列由移动边界节点直接取值，不能用最后一个固定距离列替代。

前一秒最大水分浓度为0.1500003469 kg/kg，终点降至0.1499998303 kg/kg。相较问题三，时长减少6.3453 h，约11.10%。这一差异对应“半径轨迹与物性公式同时改变”的两组模型，不能全部归因于尺寸收缩。

图3  固定半径与收缩条件下的中心和表面水分浓度

从几何上看，半径减小缩短了扩散路径；从物性上看，附录4与附录3的浓度因子及前因子不同。若要定量分离单一收缩效应，还需在完全相同物性条件下对固定半径和收缩半径进行对照计算，本文不据现有两组结果给出该因果分解。

# 九、模型检验与讨论

## 9.1 网格与时间容差检验

采用41、81、161和321个节点，比较积分通量方案得到的连续阈值事件。以烘干时长为主要输出，最后两档网格的相对差小于0.1%作为时长收敛的检查标准。表7给出实际计算值。

表7  不同径向节点数下的连续阈值事件时间

| 径向节点数 | 问题三 / h | 问题四 / h |
| --- | --- | --- |
| 41 | 57.179988 | 50.833750 |
| 81 | 57.172134 | 50.826623 |
| 161 | 57.170087 | 50.824811 |
| 321 | 57.169559 | 50.824353 |

161至321节点加密后，问题三和问题四的连续事件分别改变1.9016 s和1.6476 s，相对差分别为0.000924%和0.000900%。在321节点下，将相对容差由10⁻⁷加严至10⁻⁹并将最大步长由60 s减至30 s，问题三事件时间改变0.001010 s，整数秒终点保持一致。

时间收敛与场值收敛应分别判断。以41个公共材料坐标和所列采样时刻比较161与321节点结果，问题一、问题三和问题四的最大含水率差分别约为0.000352、0.000276和0.002120 kg/kg。这些差异说明，当前结果可以支持所报告的时长比较，但不能据此声称全部场值已经稳定到小数点后四位。

## 9.2 初始条件与离散通量检查

逐项检查了初始行、内部积分节点及每60 s插值样本。初始温度和含水率精确保存为28 ℃与2.55 kg/kg；计算浓度保持为正，未出现超出初值的实质性增长；温度保持在初值与所给环境输入形成的物理范围内。问题四逐时检查固定输出坐标是否位于当前半径以内，域外输出保留空值。

对若干时刻，将全部控制体方程相加，内部界面通量相互抵消，结果与表面通量一致，残差仅为浮点运算量级。该检查验证离散算子的通量一致性，不能替代含变物性、蒸发潜热和真实干固体质量的完整热力学守恒验证。

## 9.3 适用范围与未验证因素

题目未提供药材内部温度或含水率的实测序列，因此本文尚未完成与实验观测的直接对比。网格检验评价的是数值离散误差，不能给出一维简化、物性经验公式及边界外推带来的总模型误差。本文不为这些误差指定缺乏依据的百分比，也不据本模型推算未经测量的药效成分损失或实际生产能耗。

半径数据虽能确定几何变化，却不足以唯一确定内部应变、轴向收缩和干固体质量密度的空间变化。问题四应理解为在均匀径向收缩与有效扩散假设下的预测。若需要完整的水质量守恒模型，应补充干固体密度及变形信息，再联合建立固体连续方程与水分输运方程。

# 十、模型评价与改进

模型将四个问题统一到相同的边界输入、物性更新、界面通量和终止判据之中。控制体形式自然处理中心对称边界，浓度积分通量适用于扩散系数对含水率高度敏感的情形，材料坐标避免了收缩过程中简单截取外层节点。完整时空结果与论文六张表由同一份未舍入解生成，可以相互核对。

主要局限在于忽略轴向输运与蒸发潜热，并采用给定经验物性和环境末值外推。进一步研究可依次增加端面与潜热项，利用内部测温和含水率试验校核模型，在近表面区域采用自适应网格，并将材料变形与干固体质量守恒联合求解。工艺优化还需要独立的能耗、品质及设备约束；本研究仅提供所列假设下的状态预测与时长比较。

# 参考文献

[1] 全国大学生数学建模竞赛组委会. 2026年高教社杯全国大学生数学建模竞赛A题 药材的烘干问题及附件[Z]. 2026.

[2] CRANK J. The Mathematics of Diffusion[M]. 2nd ed. Oxford: Clarendon Press, 1975.

[3] SciPy Developers. scipy.integrate.solve_ivp[EB/OL]. SciPy Reference Guide. [2026-09-11]. https://docs.scipy.org/doc/scipy/reference/generated/scipy.integrate.solve_ivp.html.

# 附录A 程序与完整结果说明

支撑文件位于与本论文同目录的“复算附件”文件夹。recompute.py包含模型、数据读取、数值积分、收敛检验和未舍入结果生成程序；summary.json保存论文六张表及验证指标；四份result文件保存题目要求的完整输出。以下表格统计数据行数，不含表头。

表B  完整结果文件的输出约定

| 文件 | 内容 | 时间间隔 | 数据行数 |
| --- | --- | --- | --- |
| result1.xlsx | 温度与水分浓度 | 1 s | 1801 |
| result2.xlsx | 温度与水分浓度 | 1 s | 10801 |
| result3.xlsx | 水分浓度 | 60 s及终点 | 3432 |
| result4.xlsx | 水分浓度 | 60 s及终点 | 3051 |

各表A列为时间，首行为到中心的距离，径向输出间隔0.1 cm。result3和result4的最后一行可能与前一行不足60 s，以准确保留整数秒达标终点。result4中域外单元格为空，真实表面值和半径序列保存在对应JSON中，表6据该表面序列生成。

其他支撑文件包括recompute.py、export_results.mjs、make_figures.py、summary.json、plot_data.npz、manifest.json、运行说明.txt，以及result1_full.json至result4_full.json。四份JSON保存未舍入数据与表面信息；NPZ为绘图数据；manifest记录文件校验信息。

数值程序依赖 Python、NumPy、SciPy 和 openpyxl。openpyxl仅用于读取题目输入；完整数值结果首先写为JSON，配套导出程序再将其写入Excel。复算时从原始附件读取数据，不从已经填好的result表反向读取。

命令行示例：

python recompute.py --input-dir "A题目录" --output-dir "复算输出目录"

输入目录应包含“附件/附件1.xlsx”和“附件/附件2.xlsx”。若省略输出目录，结果写入脚本所在目录。数值程序的完整源码随支撑材料提供；Excel导出脚本及其运行说明同时放在复算附件中。

# 附录B 数值求解完整源程序

以下为recompute.py的完整源码。运行时使用附录A所列输入目录参数，输出目录中的JSON与摘要数据由同一次计算生成。

"""Recompute CUMCM A with conservative radial finite volumes and SciPy BDF.

Run with Python plus numpy, scipy, openpyxl. The original inputs are read only.

Temperature state is Celsius, converted to Kelvin only in Arrhenius factors.

For Q4 x=r/R(t) is a fixed material coordinate under uniform shrinkage.

No concentration clipping, explicit CFL relaxation, density weighting of C,

or duplicate mesh-advection/dilution term is used.

"""

from pathlib import Path

import argparse, json, math, time, shutil, hashlib

from scipy.special import expi

from scipy.integrate import quad

import numpy as np

import scipy

from scipy.integrate import solve_ivp

from scipy.sparse import lil_matrix

from openpyxl import load_workbook

BASE = None

WORK = None

DEST = None

ENV = None

RAD = None

HT, HM, TARGET = 25.0, 8e-7, 0.15

def read_input(name, width):

with_path = BASE / "附件" / name

wb = load_workbook(with_path, read_only=True, data_only=True)

rows = [list(r[:width]) for r in wb.active.iter_rows(values_only=True)][1:]

wb.close()

a = np.asarray(rows, dtype=float)

assert np.isfinite(a).all() and np.all(np.diff(a[:, 0]) > 0) and a[0, 0] == 0

return a

def configure(input_dir, output_dir):

global BASE, WORK, DEST, ENV, RAD

BASE = Path(input_dir).resolve()

WORK = Path(output_dir).resolve()

WORK.mkdir(parents=True, exist_ok=True)

DEST = WORK

ENV = read_input("附件1.xlsx", 3)

RAD = read_input("附件2.xlsx", 2)

assert ENV[0, 1] == 28 and RAD[0, 1] == 2 and (RAD[:, 1] > 0).all()

def phi(C, b):

return C * np.exp(-b / C) + b * expi(-b / C)

def moisture_parameters(q, T):

if q == 1:

return np.asarray(T) * 0 + 7e-9, 0.89

if q == 3:

return 2.4e-3 * np.exp(-3850 / (T + 273.15)), 0.45

if q == 4:

return 4.2e-4 * np.exp(-3850 / (T + 273.15)), 0.30

raise ValueError(q)

def radius(q, t):

return (

np.interp(t, RAD[:, 0], RAD[:, 1]) * 0.01

if q == 4

else np.asarray(t) * 0 + 0.02

)

def props(q, T, C):

if q == 1:

return (

np.full_like(C, 820 * 2600),

np.full_like(C, 0.36),

7e-9 * np.exp(-0.89 / C),

)

if q == 3:

return (

(650 + 128 * C) * (1450 + 2736 * C / (C + 1)),

0.21 + 0.38 * C / (C + 1),

2.4e-3 * np.exp(-0.45 / C) * np.exp(-3850 / (T + 273.15)),

)

if q == 4:

return (

(760 + 90 * C) * (1850 + 2150 * C / (C + 1)),

0.12 + 0.20 * C / (C + 1),

4.2e-4 * np.exp(-0.30 / C) * np.exp(-3850 / (T + 273.15)),

)

raise ValueError(q)

def model(q, n, moisture_flux="integrated"):

x = np.linspace(0, 1, n)

faces = np.r_[0, (x[:-1] + x[1:]) / 2, 1]

vol = (faces[1:] ** 2 - faces[:-1] ** 2) / 2

dx = x[1] - x[0]

def flux_div(u, a, ambient, h, R):

harmonic = 2 * a[:-1] * a[1:] / (a[:-1] + a[1:])

flux = np.zeros(n + 1)

flux[1:-1] = faces[1:-1] * harmonic * np.diff(u) / dx

flux[-1] = R * h * (ambient - u[-1])

return np.diff(flux) / (R * R * vol)

def rhs(t, y):

T, C = y[:n], y[n:]

cap, k, D = props(q, T, C)

R = float(radius(q, t))

Ta = np.interp(t, ENV[:, 0], ENV[:, 1])

Ca = np.interp(t, ENV[:, 0], ENV[:, 2])

if moisture_flux == "harmonic":

dC = flux_div(C, D, Ca, HM, R)

else:

A, b = moisture_parameters(q, (T[:-1] + T[1:]) / 2)

flux = np.zeros(n + 1)

flux[1:-1] = faces[1:-1] * A * np.diff(phi(C, b)) / dx

flux[-1] = R * HM * (Ca - C[-1])

dC = np.diff(flux) / (R * R * vol)

return np.r_[flux_div(T, k, Ta, HT, R) / cap, dC]

mask = lil_matrix((2 * n, 2 * n), dtype=int)

for i in range(n):

for j in range(max(0, i - 1), min(n, i + 2)):

mask[i, j] = mask[i, n + j] = mask[n + i, j] = mask[n + i, n + j] = 1

def event(t, y):

return np.max(y[n:]) - TARGET

event.terminal = True

event.direction = -1

return x, vol, rhs, mask.tocsr(), event

class Run:

def __init__(self, q, n=81, rtol=1e-7, max_step=60, moisture_flux="integrated"):

start = time.perf_counter()

self.q, self.n, self.rtol = q, n, rtol

self.moisture_flux = moisture_flux

self.x, self.vol, rhs, sparsity, event = model(q, n, moisture_flux)

self.rhs = rhs

y0 = np.r_[np.full(n, 28.0), np.full(n, 2.55)]

kw = dict(

method="BDF",

rtol=rtol,

atol=np.r_[np.full(n, rtol * 0.1), np.full(n, rtol * 0.001)],

max_step=max_step,

jac_sparsity=sparsity,

dense_output=True,

)

self.main = solve_ivp(

rhs,

(0, 1800 if q == 1 else 14 * 86400),

y0,

events=None if q == 1 else event,

**kw,

)

assert self.main.success, self.main.message

self.tail = None

if q != 1:

assert len(self.main.t_events[0]) == 1, "Threshold not reached in 14 days"

self.event_s = float(self.main.t_events[0][0])

self.end_s = int(math.floor(self.event_s) + 1)

self.tail = solve_ivp(

rhs, (self.event_s, self.end_s), self.main.y[:, -1], **kw

)

assert self.tail.success

self.end_max = float(self.tail.y[n:, -1].max())

self.prev_max = float(self.main.sol(self.end_s - 1)[n:].max())

assert self.end_max < TARGET and self.prev_max >= TARGET, (

self.end_max,

self.prev_max,

)

else:

self.event_s = None

self.end_s = 1800

self.elapsed = time.perf_counter() - start

self.checks = self.validate()

print(json.dumps(self.metadata()), flush=True)

def sample(self, t):

t = np.atleast_1d(t).astype(float)

a = np.empty((2 * self.n, len(t)))

inside = t <= self.main.t[-1]

if inside.any():

a[:, inside] = self.main.sol(t[inside])

if (~inside).any():

assert self.tail is not None and t.max() <= self.end_s

a[:, ~inside] = self.tail.sol(t[~inside])

return a

def physical(self, t, cm):

t = np.atleast_1d(t).astype(float)

cm = np.asarray(cm)

a = self.sample(t)

rr = radius(self.q, t) * 100

T, C = np.full((len(t), len(cm)), np.nan), np.full((len(t), len(cm)), np.nan)

for j in range(len(t)):

valid = cm <= rr[j] + 1e-12

T[j, valid] = np.interp(cm[valid] / rr[j], self.x, a[: self.n, j])

C[j, valid] = np.interp(cm[valid] / rr[j], self.x, a[self.n :, j])

return T, C

def validate(self):

states = self.main.y if self.tail is None else np.c_[self.main.y, self.tail.y]

T, C = states[: self.n], states[self.n :]

tcheck = np.unique(np.r_[np.arange(0, self.end_s + 1, 60), self.end_s])

sampled = self.sample(tcheck)

Tmin = float(min(T.min(), sampled[: self.n].min()))

Tmax = float(max(T.max(), sampled[: self.n].max()))

Cmin = float(min(C.min(), sampled[self.n :].min()))

Cmax = float(max(C.max(), sampled[self.n :].max()))

initial_ok = np.array_equal(

self.sample([0])[:, 0], np.r_[np.full(self.n, 28.0), np.full(self.n, 2.55)]

)

bounds_ok = (

Tmin >= min(28, float(ENV[:, 1].min())) - 1e-6

and Tmax <= max(28, float(ENV[:, 1].max())) + 1e-6

and Cmin > 0

and Cmax <= 2.55 + 1e-8

)

assert initial_ok and bounds_ok, (Tmin, Tmax, Cmin, Cmax)

conservation = []

for tt in [100.0, min(1800.0, self.end_s), float(self.end_s)]:

yy = self.sample([tt])[:, 0]

dy = self.rhs(tt, yy)

cap, _, _ = props(self.q, yy[: self.n], yy[self.n :])

R = float(radius(self.q, tt))

lhsT = float(np.sum(self.vol * R**2 * cap * dy[: self.n]))

rhsT = (

R * HT * (float(np.interp(tt, ENV[:, 0], ENV[:, 1])) - yy[self.n - 1])

)

lhsC = float(np.sum(self.vol * R**2 * dy[self.n :]))

rhsC = R * HM * (float(np.interp(tt, ENV[:, 0], ENV[:, 2])) - yy[-1])

conservation.append(

{

"time_s": tt,

"heat_residual": float(lhsT - rhsT),

"moisture_residual": float(lhsC - rhsC),

}

)

assert abs(lhsT - rhsT) < 1e-10 and abs(lhsC - rhsC) < 1e-18

return dict(

initial_exact=bool(initial_ok),

physical_bounds=bool(bounds_ok),

temperature_min=Tmin,

temperature_max=Tmax,

moisture_min=Cmin,

moisture_max=Cmax,

discrete_flux_balance=conservation,

inspection="accepted internal steps and 60-second dense samples; no clipping",

)

def metadata(self):

d = dict(

problem=self.q,

nodes=self.n,

moisture_flux=self.moisture_flux,

rtol=self.rtol,

end_s=self.end_s,

end_h=self.end_s / 3600,

event_s=self.event_s,

event_h=None if self.event_s is None else self.event_s / 3600,

nfev=self.main.nfev,

njev=self.main.njev,

nlu=self.main.nlu,

accepted_steps=len(self.main.t) - 1,

elapsed_s=self.elapsed,

checks=self.checks,

)

if self.q != 1:

d.update(

endpoint_max_C=self.end_max,

previous_second=self.end_s - 1,

previous_second_max_C=self.prev_max,

)

return d

def clean(v):

if isinstance(v, np.ndarray):

return clean(v.tolist())

if isinstance(v, (np.floating, float)):

return None if not np.isfinite(v) else float(v)

if isinstance(v, (np.integer,)):

return int(v)

if isinstance(v, dict):

return {str(k): clean(x) for k, x in v.items()}

if isinstance(v, (list, tuple)):

return [clean(x) for x in v]

return v

def write_json(name, data):

text = json.dumps(clean(data), ensure_ascii=False, indent=None, allow_nan=False)

(WORK / name).write_text(text, encoding="utf-8")

if WORK != DEST:

shutil.copy2(WORK / name, DEST / name)

def table(run, times, cm, field):

T, C = run.physical(times, cm)

a = run.sample(times)

d = dict(

time_s=np.asarray(times),

time_h=np.asarray(times) / 3600,

radius_positions_cm=cm,

values=T if field == "temperature" else C,

units="degC" if field == "temperature" else "kg/kg",

)

if run.q == 4:

d.update(

surface_values=a[run.n - 1 if field == "temperature" else -1],

surface_radius_cm=radius(4, np.asarray(times)) * 100,

)

return d

def complete(run, times, name):

cm = np.linspace(0, 2, 21)

T, C = run.physical(times, cm)

a = run.sample(times)

data = dict(

time_s=times,

radius_positions_cm=cm,

temperature_C=T,

moisture_kg_kg=C,

surface_temperature_C=a[run.n - 1],

surface_moisture_kg_kg=a[-1],

surface_radius_cm=radius(run.q, np.asarray(times)) * 100,

metadata=run.metadata(),

)

write_json(name, data)

def compare(coarse, fine):

times = np.unique(

np.r_[

0,

100,

300,

600,

900,

1200,

1500,

1800,

np.arange(21600, min(coarse.end_s, fine.end_s), 21600),

min(coarse.end_s, fine.end_s),

]

)

vals = []

for run in (coarse, fine):

a = run.sample(times)

grid = np.linspace(0, 1, 41)

vals.append(

np.stack(

[

np.stack(

[

np.interp(grid, run.x, a[j * run.n : (j + 1) * run.n, i])

for i in range(len(times))

]

)

for j in (0, 1)

]

)

)

dif = np.abs(vals[0] - vals[1])

d = dict(

coarse_nodes=coarse.n,

fine_nodes=fine.n,

coarse_rtol=coarse.rtol,

fine_rtol=fine.rtol,

max_temperature_difference=float(dif[0].max()),

max_moisture_difference=float(dif[1].max()),

comparison_times_s=times.tolist(),

comparison_grid="41 common normalized material coordinates x=0..1",

)

if coarse.event_s is not None:

d.update(

event_time_difference_s=coarse.event_s - fine.event_s,

relative_event_difference=abs(coarse.event_s - fine.event_s) / fine.event_s,

integer_end_difference_s=coarse.end_s - fine.end_s,

)

return d

def outputs(summary, runs):

q1, q3, q4 = runs[1], runs[3], runs[4]

times1 = np.array([100, 300, 600, 900, 1200, 1500, 1800])

times2 = np.arange(1800, 10801, 1800)

cm = [0, 0.5, 1, 1.5, 2]

for num, run, ts, field in [

(1, q1, times1, "temperature"),

(2, q1, times1, "moisture"),

(3, q3, times2, "temperature"),

(4, q3, times2, "moisture"),

]:

summary["tables"][str(num)] = table(run, ts, cm, field)

for num, run in [(5, q3), (6, q4)]:

ts = np.r_[np.arange(21600, run.end_s, 21600), run.end_s]

summary["tables"][str(num)] = table(

run, ts, [0, 0.5, 1, 1.5] if run.q == 4 else cm, "moisture"

)

summary["baselines"]["2"] = {

"source_problem": 3,

"end_s": 10800,

"end_h": 3,

"comment": "Same PDE and initial conditions as Q3, exact dense-output prefix of that integration.",

}

summary["radius_reduction_at_q4_end"] = 1 - float(radius(4, q4.end_s)) / 0.02

summary["q4_vs_q3_duration_ratio"] = q4.end_s / q3.end_s

write_json("summary.json", summary)

complete(q1, np.arange(1801), "result1_full.json")

complete(q3, np.arange(10801), "result2_full.json")

for run in [q3, q4]:

complete(

run,

np.r_[np.arange(0, run.end_s, 60), run.end_s],

f"result{run.q}_full.json",

)

plots = {}

for q, run in runs.items():

tt = np.unique(

np.r_[np.arange(0, run.end_s + 1, 10 if q == 1 else 60), run.end_s]

)

aa = run.sample(tt)

plots.update(

{

f"q{q}_time_s": tt,

f"q{q}_center_T": aa[0],

f"q{q}_surface_T": aa[run.n - 1],

f"q{q}_center_C": aa[run.n],

f"q{q}_surface_C": aa[-1],

f"q{q}_radius_cm": radius(q, tt) * 100,

}

)

rt = (

times1

if q == 1

else np.unique(

np.r_[1800, 10800, np.arange(21600, run.end_s, 21600), run.end_s]

)

)

a = run.sample(rt)

plots.update(

{

f"q{q}_profile_times_s": rt,

f"q{q}_profile_radius_cm": radius(q, rt)[:, None]

* run.x[None, :]

* 100,

f"q{q}_profile_T": a[: run.n].T,

f"q{q}_profile_C": a[run.n :].T,

}

)

plots["environment"] = ENV

plots["radius_input"] = RAD

np.savez_compressed(WORK / "plot_data.npz", **plots)

if WORK != DEST:

shutil.copy2(WORK / "plot_data.npz", DEST / "plot_data.npz")

def verify_phi():

checks = []

for b in [0.89, 0.45, 0.30]:

for C in [0.01963, 0.05, 0.15, 0.5, 2.55]:

h = C * 1e-4

deriv = (

-phi(C + 2 * h, b)

+ 8 * phi(C + h, b)

- 8 * phi(C - h, b)

+ phi(C - 2 * h, b)

) / (12 * h)

exact = np.exp(-b / C)

rel = abs(deriv - exact) / exact

assert rel < 1e-7, (b, C, rel)

checks.append(

dict(

b=b,

C=C,

numerical_derivative=float(deriv),

exact_derivative=float(exact),

relative_error=float(rel),

)

)

for lo, hi in [(0.01963, 0.05), (0.05, 0.15), (0.15, 2.55)]:

v = phi(hi, b) - phi(lo, b)

integ, err = quad(

lambda c: np.exp(-b / c), lo, hi, epsabs=1e-15, epsrel=1e-12

)

assert abs(v - integ) < 1e-12 * max(abs(integ), 1e-10)

checks.append(

dict(

b=b,

interval=[lo, hi],

phi_difference=float(v),

quad=float(integ),

absolute_error=float(abs(v - integ)),

)

)

return checks

def validate_outputs():

report = {}

for q in [1, 2, 3, 4]:

r = json.loads((WORK / f"result{q}_full.json").read_text(encoding="utf-8"))

ts = np.asarray(r["time_s"])

T = np.asarray(r["temperature_C"], dtype=float)

C = np.asarray(r["moisture_kg_kg"], dtype=float)

assert T.shape == C.shape == (len(ts), 21)

assert np.array_equal(T[0], np.full(21, 28.0)) and np.array_equal(

C[0], np.full(21, 2.55)

)

assert np.all(np.diff(ts) > 0)

if q in (1, 2):

assert np.all(np.diff(ts) == 1) and ts[-1] == (1800 if q == 1 else 10800)

else:

assert np.all(np.diff(ts)[:-1] == 60) and 0 < np.diff(ts)[-1] <= 60

if q == 4:

expected = (

np.asarray(r["radius_positions_cm"])[None, :]

> np.asarray(r["surface_radius_cm"])[:, None] + 1e-12

)

assert np.array_equal(np.isnan(C), expected) and np.array_equal(

np.isnan(T), expected

)

else:

assert np.isfinite(T).all() and np.isfinite(C).all()

report[str(q)] = dict(

rows=len(ts),

columns=21,

initial_row_exact=True,

time_grid_valid=True,

outside_domain_null_count=int(np.isnan(C).sum()),

)

return report

def main():

parser = argparse.ArgumentParser(description=__doc__)

parser.add_argument(

"--input-dir",

type=Path,

required=True,

help="A problem directory containing the attachments subdirectory",

)

parser.add_argument(

"--output-dir", type=Path, default=Path(__file__).resolve().parent

)

args = parser.parse_args()

configure(args.input_dir, args.output_dir)

summary = dict(

method=dict(

spatial="conservative radial finite volume; endpoints included; midpoint faces; harmonic thermal conductivity; nonlinear integrated moisture flux",

moisture_face_flux="x_face*A(T_face)*(Phi(C_right)-Phi(C_left))/dx, T_face=(T_left+T_right)/2",

phi="Phi(C)=C*exp(-b/C)+b*Ei(-b/C); derivative exp(-b/C)",

time="solve_ivp BDF with sparse Jacobian pattern and dense output",

nodes={},

rtol=1e-7,

temperature_atol=1e-8,

moisture_atol=1e-10,

max_step_s=60,

initial_temperature_C=28,

initial_moisture_kg_kg=2.55,

heat_transfer_coefficient=HT,

mass_transfer_coefficient=HM,

threshold=TARGET,

threshold_rule="first integer second strictly after continuous max(C)=0.15 crossing, verified against preceding integer second",

shrinkage="uniform material shrinkage x=r/R(t); fixed material mesh, effective dry-basis C has no separate dilution term",

interpolation="linear between input data; endpoint values outside input range",

scope="radial-only; ignore axial transport and evaporation latent heat; Q2/Q3/Q4 retain HT=25 and HM=8e-7 because appendices 3/4 provide no replacement values",

versions=dict(numpy=np.__version__, scipy=scipy.__version__),

),

inputs=dict(

environment_rows=len(ENV),

environment_first=ENV[0],

environment_last=ENV[-1],

radius_rows=len(RAD),

radius_first=RAD[0],

radius_last=RAD[-1],

),

baselines={},

tables={},

convergence={},

time_tolerance={},

integrated_flux_verification=verify_phi(),

spatial_converged=False,

verification_complete=False,

)

runs = {}

meshes = {}

for q in [1, 3, 4]:

meshes[q] = [Run(q, n=n) for n in [41, 81, 161, 321]]

comp = compare(meshes[q][-2], meshes[q][-1])

if q != 1 and comp["relative_event_difference"] >= 0.001:

meshes[q].append(Run(q, n=641))

runs[q] = meshes[q][-1]

summary["baselines"][str(q)] = runs[q].metadata()

summary["method"]["nodes"][str(q)] = runs[q].n

summary["convergence"][str(q)] = {

"runs": [r.metadata() for r in meshes[q]],

"comparisons": [

compare(a, b) for a, b in zip(meshes[q][:-1], meshes[q][1:])

],

}

last = summary["convergence"][str(q)]["comparisons"][-1]

passed = last.get("relative_event_difference", 0) < 0.001

summary["convergence"][str(q)]["spatial_converged"] = passed if q != 1 else None

summary["convergence"][str(q)][

"criterion"

] = "consecutive event-time relative difference below 0.001 for drying endpoints; Q1 reports actual field differences without claiming four-decimal field convergence"

write_json("summary.json", summary)

summary["spatial_converged"] = all(

summary["convergence"][str(q)]["spatial_converged"] for q in [3, 4]

)

outputs(summary, runs)

print("INTEGRATED_OUTPUTS_READY", flush=True)

summary["harmonic_diagnostic"] = {}

for q in [1, 3, 4]:

hs = [Run(q, n=n, moisture_flux="harmonic") for n in [321, 641]]

summary["harmonic_diagnostic"][str(q)] = {

"runs": [r.metadata() for r in hs],

"321_vs_641": compare(*hs),

"harmonic641_vs_final_integrated": compare(hs[-1], runs[q]),

}

write_json("summary.json", summary)

strict = Run(3, n=runs[3].n, rtol=1e-9, max_step=30)

summary["time_tolerance"]["3"] = {

"run": strict.metadata(),

"base_vs_strict": compare(runs[3], strict),

}

summary["verification_complete"] = True

summary["output_validation"] = validate_outputs()

summary["numerical_limitations"] = [

"Discrete flux-balance identities verify cancellation of internal numerical face fluxes only. They do not establish conservation of physical total water mass or total energy for moisture-dependent density/capacity or imposed material shrinkage.",

"No experimental drying profiles are supplied for calibration or validation. Radius observations are imposed as model input.",

"The reported endpoint criterion is successive-grid agreement below 0.1%, not rigorous discretization-error bounds or four-decimal accuracy of every field entry.",

"Q4 results depend on the stated uniform material shrinkage effective dry-basis model and prescribed radius extension.",

]

write_json("summary.json", summary)

src = Path(__file__).resolve()

if src != WORK / src.name:

shutil.copy2(src, WORK / src.name)

manifest = {

p.name: dict(

bytes=p.stat().st_size, sha256=hashlib.sha256(p.read_bytes()).hexdigest()

)

for p in WORK.iterdir()

if p.is_file()

and p.suffix in (".json", ".npz", ".py")

and p.name != "manifest.json"

}

write_json("manifest.json", manifest)

print("ALL_COMPLETE", flush=True)

if __name__ == "__main__":

main()

# 附录C 结果表导出完整源程序

以下为export_results.mjs的完整源码。使用Node.js及@oai/artifact-tool读取四份完整JSON并导出对应Excel结果，使用方法见配套说明。

import fs from "node:fs/promises";

import path from "node:path";

import { createRequire } from "node:module";

import { fileURLToPath, pathToFileURL } from "node:url";

// Dependency: @oai/artifact-tool 2.8.58 or later.

// Usage: node export_results.mjs [data-directory]

// The optional directory must contain result1_full.json through result4_full.json.

// When omitted, the script reads and writes files beside export_results.mjs.

async function loadArtifactTool() {

try {

return await import("@oai/artifact-tool");

} catch (directError) {

try {

const requireFromCwd = createRequire(path.join(process.cwd(), "package.json"));

const resolved = requireFromCwd.resolve("@oai/artifact-tool");

return await import(pathToFileURL(resolved).href);

} catch {

throw new Error(

"Cannot load @oai/artifact-tool. Install or expose it in node_modules before running this script.",

{ cause: directError },

);

}

}

}

const { SpreadsheetFile, Workbook } = await loadArtifactTool();

const scriptDir = path.dirname(fileURLToPath(import.meta.url));

const dataDir = path.resolve(process.argv[2] ?? scriptDir);

const chineseFont = "SimSun";

const latinFont = "Consolas";

const workbookConfigs = [

{

index: 1,

sheets: [

["\u6e29\u5ea6", "temperature_C"],

["\u6c34\u5206\u6d53\u5ea6", "moisture_kg_kg"],

],

},

{

index: 2,

sheets: [

["\u6e29\u5ea6", "temperature_C"],

["\u6c34\u5206\u6d53\u5ea6", "moisture_kg_kg"],

],

},

{ index: 3, sheets: [["Sheet1", "moisture_kg_kg"]] },

{ index: 4, sheets: [["Sheet1", "moisture_kg_kg"]] },

];

function assert(condition, message) {

if (!condition) throw new Error(message);

}

function maxIgnoringNull(values) {

let maximum = -Infinity;

for (const value of values) {

if (value !== null && value > maximum) maximum = value;

}

return maximum;

}

function validateSource(source, index) {

assert(source.time_s.length > 0, `result${index}: empty time axis`);

assert(source.radius_positions_cm.length === 21, `result${index}: expected 21 radius positions`);

assert(

source.radius_positions_cm.every((value, i) => Math.abs(value - i / 10) < 1e-12),

`result${index}: radius positions must be 0.0 to 2.0 cm in 0.1 cm steps`,

);

for (const key of ["temperature_C", "moisture_kg_kg"]) {

assert(source[key].length === source.time_s.length, `result${index}/${key}: row count mismatch`);

assert(source[key].every((row) => row.length === 21), `result${index}/${key}: column count mismatch`);

}

if (index >= 3) {

assert(maxIgnoringNull(source.moisture_kg_kg.at(-1)) < 0.15, `result${index}: endpoint max C is not below 0.15`);

}

}

function populateSheet(sheet, source, dataKey) {

const rowCount = source.time_s.length;

const lastRow = rowCount + 1;

const matrix = source[dataKey];

sheet.getRange("A1").write([["\u65f6\u95f4", ...source.radius_positions_cm]]);

sheet.getRange("A2").write(source.time_s.map((time, i) => [time, ...matrix[i]]));

const usedRange = sheet.getRange(`A1:V${lastRow}`);

usedRange.format.font = { name: latinFont, size: 10, color: "#222222" };

usedRange.format.verticalAlignment = "center";

sheet.getRange("A1:V1").format = {

fill: "#E7E6E6",

font: { name: latinFont, size: 10, bold: true, color: "#222222" },

horizontalAlignment: "center",

verticalAlignment: "center",

borders: { bottom: { style: "thin", color: "#A6A6A6" } },

};

sheet.getRange("A1").format.font = { name: chineseFont, size: 10, bold: true, color: "#222222" };

sheet.getRange(`A2:A${lastRow}`).format.numberFormat = "0";

sheet.getRange("B1:V1").format.numberFormat = "0.0";

sheet.getRange(`B2:V${lastRow}`).format.numberFormat = "0.0000";

sheet.getRange(`A1:A${lastRow}`).format.columnWidth = 12;

sheet.getRange(`B1:V${lastRow}`).format.columnWidth = 11;

sheet.getRange("A1:V1").format.rowHeight = 20;

sheet.freezePanes.freezeRows(1);

sheet.freezePanes.freezeColumns(1);

}

for (const config of workbookConfigs) {

const jsonPath = path.join(dataDir, `result${config.index}_full.json`);

const source = JSON.parse(await fs.readFile(jsonPath, "utf8"));

validateSource(source, config.index);

const workbook = Workbook.create();

for (const [sheetName, dataKey] of config.sheets) {

populateSheet(workbook.worksheets.add(sheetName), source, dataKey);

}

workbook.recalculate();

const output = await SpreadsheetFile.exportXlsx(workbook);

const outputPath = path.join(dataDir, `result${config.index}.xlsx`);

await output.save(outputPath);

// artifact-tool may emit an inspection sidecar during export; it is not a deliverable.

await fs.rm(`${outputPath}.inspect.ndjson`, { force: true });

console.log(`Created ${outputPath}`);

}

# 附录D 绘图完整源程序

以下程序根据同目录plot_data.npz生成本文图1至图3。运行依赖NumPy和Matplotlib。中文为宋体，英文和数字为Consolas。

from pathlib import Path

import json, numpy as np

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

from matplotlib import font_manager as fm

DATA = Path(__file__).resolve().parent

OUT = DATA / "figures"

OUT.mkdir(exist_ok=True)

for f in [r"C:\Windows\Fonts\simsun.ttc", r"C:\Windows\Fonts\consola.ttf"]:

fm.fontManager.addfont(f)

plt.rcParams.update(

{

"font.family": ["Consolas", "SimSun"],

"axes.unicode_minus": False,

"font.size": 10,

"axes.labelsize": 10,

"xtick.labelsize": 9,

"ytick.labelsize": 9,

"legend.fontsize": 9,

"axes.spines.top": False,

"axes.spines.right": False,

"axes.linewidth": 0.7,

"lines.linewidth": 1.6,

"savefig.facecolor": "white",

}

)

z = np.load(DATA / "plot_data.npz")

env = z["environment"]

rad = z["radius_input"]

blue = "#205F85"

orange = "#B45D35"

gray = "#555555"

fig, ax = plt.subplots(1, 2, figsize=(9.6, 3.1))

a = ax[0]

twin = a.twinx()

l1 = a.plot(env[:, 0] / 3600, env[:, 1], c=blue, label="烘房温度")

l2 = twin.plot(env[:, 0] / 3600, env[:, 2], c=orange, label="烘房水分浓度")

a.set(xlabel="时间 / h", ylabel="温度 / ℃", title="(a) 烘房环境")

twin.set_ylabel("水分浓度 / (kg/kg)")

a.legend(l1 + l2, [l.get_label() for l in l1 + l2], loc="lower right", frameon=False)

a.grid(alpha=0.15)

ax[1].plot(rad[:, 0] / 3600, rad[:, 1], c=blue)

ax[1].set(xlabel="时间 / h", ylabel="半径 / cm", title="(b) 半径观测")

ax[1].grid(alpha=0.15)

fig.tight_layout(w_pad=2.1)

fig.savefig(OUT / "inputs.png", dpi=260)

plt.close(fig)

fig, ax = plt.subplots(2, 2, figsize=(9.6, 5.5))

for row, q in enumerate([1, 3]):

mask = z[f"q{q}_time_s"] <= (1800 if q == 1 else 10800)

ts = z[f"q{q}_time_s"][mask] / 3600

for col, field in enumerate(["T", "C"]):

a = ax[row, col]

a.plot(ts, z[f"q{q}_center_{field}"][mask], c=blue, label="中心")

a.plot(ts, z[f"q{q}_surface_{field}"][mask], c=orange, label="表面")

a.set(

xlabel="时间 / h",

ylabel="温度 / ℃" if field == "T" else "水分浓度 / (kg/kg)",

title=f"({chr(97+row*2+col)}) 问题" + ("一" if q == 1 else "二"),

)

a.grid(alpha=0.15)

a.legend(frameon=False, loc="best")

fig.tight_layout(h_pad=1.3, w_pad=2.0)

fig.savefig(OUT / "short_term.png", dpi=260)

plt.close(fig)

fig, ax = plt.subplots(1, 2, figsize=(9.6, 3.2))

for a, q, lab in zip(ax, [3, 4], ["(a) 固定半径", "(b) 给定半径收缩"]):

ts = z[f"q{q}_time_s"] / 3600

a.plot(ts, z[f"q{q}_center_C"], c=blue, label="中心")

a.plot(ts, z[f"q{q}_surface_C"], c=orange, label="表面")

a.axhline(0.15, c=gray, ls="--", lw=1, label="达标阈值")

a.set(xlabel="时间 / h", ylabel="水分浓度 / (kg/kg)", title=lab, ylim=(0, 2.7))

a.grid(alpha=0.15)

a.legend(frameon=False, loc="upper right")

a.plot(ts[-1], z[f"q{q}_center_C"][-1], "o", c=blue, ms=3.5)

fig.tight_layout(w_pad=2.0)

fig.savefig(OUT / "drying.png", dpi=260)

plt.close(fig)

print("Saved figures", [p.name for p in OUT.glob("*.png")])